ACL (Access Control List) Lab Report

1. Introduction

ACL (Access Control List) is a network security feature used on Cisco routers to control traffic flow by allowing or denying packets based on specific rules.

ACLs are used to improve network security and control access between different networks.

2. Types of ACL

Standard ACL

Filters traffic based only on the source IP address.

Uses numbers from:

1 - 99

1300 - 1999

Example:

access-list 10 permit 192.168.1.0 0.0.0.255

Extended ACL

Filters traffic based on:

Source IP

Destination IP

Protocol (TCP, UDP, ICMP)

Port number

Uses numbers:

100 - 199

2000 - 2699

Example:

access-list 100 deny tcp 192.168.1.0 0.0.0.255 any eq 80

3. Objective

Control communication between networks.

Allow specific traffic.

Block unwanted traffic.

Apply security rules on router interfaces.

4. Devices Used

2 Routers

1 Switch

PCs

5. Standard ACL Configuration

Requirement:

Block PC1 network from accessing another network.

Network:

192.168.1.0/24

Configuration

Create ACL

Command:

access-list 10 deny 192.168.1.0 0.0.0.255

Explanation:

Creates ACL number 10.

Denies traffic from network 192.168.1.0.

Wildcard mask identifies the network range.

Allow Other Traffic

Command:

access-list 10 permit any

Explanation: Allows all other networks.

Apply ACL to Interface

Command:

interface g0/0

ip access-group 10 in

Explanation: Applies ACL to incoming traffic on the interface.

6. Extended ACL Configuration

Requirement:

Block HTTP traffic but allow other traffic.

Create Extended ACL

Command:

access-list 100 deny tcp 192.168.1.0 0.0.0.255 any eq 80

Explanation:

Blocks TCP traffic.

Blocks HTTP port 80.

Applies only to source network.

Allow Remaining Traffic:

access-list 100 permit ip any any

Explanation: Allows all other IP traffic.

Apply ACL:

interface g0/1

ip access-group 100 out

7. Verification

Show ACL rules:

show access-lists

Shows:

Permit rules

Deny rules

Match counters

Check interface:

show ip interface

Shows ACL applied on interface.

8. Result

ACL was successfully configured to control network traffic and improve security by filtering unwanted communication.