OSPF Routing Protocol Lab Report

1. Introduction

OSPF (Open Shortest Path First) is a link-state routing protocol that uses the Shortest Path First algorithm to calculate the best route.

It is widely used in enterprise networks because of its scalability and fast convergence.

2. Objective

Configure OSPF between routers.

Establish neighbor relationships.

Exchange routing information.

Test connectivity between networks.

3. Devices Used

3 Cisco Routers

PCs

4. Configuration Explanation

Enable OSPF

Command:

router ospf 1

Explanation: Starts an OSPF routing process with process ID 1.

Advertise Networks

Command:

network 192.168.1.0 0.0.0.255 area 0

Explanation: Adds the network to OSPF and specifies:

Wildcard Mask

Area Number

Area 0

Explanation: Area 0 is the backbone area in OSPF where all areas must connect.

5. Verification

Command:

show ip ospf neighbor

Checks OSPF neighbor relationships.

Command:

show ip route

OSPF routes appear with: O

Result:

OSPF successfully exchanged routes between routers.