# RIP Routing Protocol Lab Report

# 1. Introduction

# RIP (Routing Information Protocol) is a distance-vector routing protocol used to exchange routing information between routers. It selects the best path based on hop count.

# 2. Objective

# Configure RIP Version 2.

# Enable communication between different networks.

# Verify routing table updates.

# 3. Devices Used

# 3 Cisco Routers

# 2 PCs

# 4. Configuration Explanation

# Enable RIP

# Command:

# router rip

# Explanation: Used to enable RIP routing process on the router.

# Select RIP Version 2

# Command:

# version 2

# Explanation: Changes RIP from version 1 to version 2 because RIP v2 supports:

# VLSM

# CIDR

# Authentication

# Add Networks

# Command:

# network 192.168.1.0

# network 10.0.12.0

# Explanation: Advertises directly connected networks to neighboring routers.

# Disable Auto Summary

# Command:

# no auto-summary

# Explanation: Prevents automatic summarization and allows the router to advertise subnet information correctly.

# Verification

# Command:

# show ip route

# Explanation: Displays learned routes. RIP routes appear with symbol: R

# Result:

# Communication between all networks was successfully achieved using RIP.